Antes que nada, si lees esto Consor, hay comandos coloque una sola vez, porque hay muchos que es escribir literalmente lo mismo pero cambiar las ips, por eso algunas veces subí solo lo de una sucursal.

enable configure terminal vlan 10 name MANAGEMENT_NET exit vlan 20 name STAFF_NET exit vlan 30 name WIFI_NET exit vlan 40 name SERVERS_NET exit vlan 50 name GUEST_NET end write memory interface range fastEthernet 0/23-24 switchport access vlan 40 no shutdown exit interface fastEthernet 0/1 switchport access vlan 30 no shutdown exit configure terminal ! 1. Configurar la interfaz física (Puerta de enlace) interface GigabitEthernet0/0/0 no ip address no shutdown exit ! 2. Configuración de Subinterfaces para cada VLAN interface gigabitEthernet 0/0/0.10 encapsulation dot1Q 10 ip address 172.31.0.1 255.255.255.224 no shutdown exit ! 3. Configuración de Subinterfaces para STAFF_NET interface gigabitEthernet 0/0/0.20 encapsulation dot1Q 20 ip address 172.31.0.33 255.255.255.224 no shutdown exit ! 4. Configuración de Subinterfaces para WIFI_NET (Corrección de máscara y dirección) interface gigabitEthernet 0/0/0.30 encapsulation dot1Q 30 ip address 172.31.0.65 255.255.255.240 no shutdown exit configure terminal ! Excluir direcciones de Gateway y estáticas (ej. servidores) ip dhcp excluded-address 172.31.0.1 172.31.0.3 ip dhcp excluded-address 172.31.0.33 172.31.0.35 ip dhcp excluded-address 172.31.0.65 172.31.0.67 ip dhcp excluded-address 172.31.0.81 172.31.0.83 ip dhcp excluded-address 172.31.0.97 172.31.0.99 ! Pool MANAGEMENT_NET (VLAN 10) ip dhcp pool MANAGEMENT_NET network 172.31.0.0 255.255.255.224 default-router 172.31.0.1 dns-server 8.8.8.8 exit ! Pool STAFF_NET (VLAN 20) ip dhcp pool STAFF_NET network 172.31.0.32 255.255.255.224 default-router 172.31.0.33 dns-server 8.8.8.8 exit ! Pool WIFI_NET (VLAN 30) ip dhcp pool WIFI_NET network 172.31.0.64 255.255.255.240 default-router 172.31.0.65 dns-server 8.8.8.8 exit ! Pool SERVERS_NET (VLAN 40) ip dhcp pool SERVERS_NET network 172.31.0.80 255.255.255.240 default-router 172.31.0.81 dns-server 8.8.8.8 exit ! Pool GUEST_NET (VLAN 50) ip dhcp pool GUEST_NET network 172.31.0.96 255.255.255.240 default-router 172.31.0.97 dns-server 8.8.8.8 exit configure terminal ! Excluir direcciones (debe coincidir con la red 192.168.100.x) ip dhcp excluded-address 192.168.100.1 192.168.100.3 ip dhcp excluded-address 192.168.100.33 192.168.100.35 ip dhcp excluded-address 192.168.100.65 192.168.100.67 ip dhcp excluded-address 192.168.100.81 192.168.100.83 ip dhcp excluded-address 192.168.100.97 192.168.100.99 ! Pool MANAGEMENT_NET ip dhcp POOL MANAGEMENT_NET dns-server 8.8.8.8 network 192.168.100.0 255.255.255.224 default-router 192.168.100.1 exit ! Pool STAFF_NET ip dhcp POol STAFF_NET dns-server 8.8.8.8 network 192.168.100.32 255.255.255.224 default-router 192.168.100.33 exit ! Pool WIFI_NET ip dhcp pool WIFI_NET dns-server 8.8.8.8 network 192.168.100.64 255.255.255.240 default-router 192.168.100.65 exit ! Pool SERVERS_NET ip dhcp pool SERVERS_NET dns-server 8.8.8.8 network 192.168.100.80 255.255.255.240 default-router 192.168.100.81 exit ! Pool GUEST_NET ip dhcp pool GUEST_NET network 192.168.100.96 255.255.255.240 default-router 192.168.100.97 exit end configure terminal interface GigabitEthernet0/0/0 no ip address no shutdown exit ! interface GigabitEthernet0/0/0.10 encapsulation dot1Q 10 ip address 203.0.113.1 255.255.255.224 no shutdown exit ! interface GigabitEthernet0/0/0.20 encapsulation dot1Q 20 ip address 203.0.113.33 255.255.255.224 no shutdown exit ! interface GigabitEthernet0/0/0.30 encapsulation dot1Q 30 ip address 203.0.113.65 255.255.255.224 no shutdown exit ! interface GigabitEthernet0/0/0.40 encapsulation dot1Q 40 ip address 203.0.113.97 255.255.255.224 no shutdown exit ! interface GigabitEthernet0/0/0.50 encapsulation dot1Q 50 ip address 203.0.113.129 255.255.255.224 no shutdown exit ### Configuración SR-1 configure terminal router rip version 2 no auto-summary network 172.31.0.0 network 11.0.0.0 network 44.0.0.0 ### Configuración SR-2 configure terminal router rip version 2 no auto-summary network 203.0.113.0 network 11.0.0.0 network 44.0.0.0 ### Configuración SR-3 configure terminal router rip version 2 no auto-summary network 192.168.100.0 network 11.0.0.0 network 44.0.0.0 ### Configuración SR-4 configure terminal router rip version 2 no auto-summary network 198.51.100.0 network 11.0.0.0 ! Las reglas de la ACL 198 (Source 198.51.100.x) access-list 198 permit ip 198.51.100.96 0.0.0.31 any access-list 198 permit ip any 172.31.0.96 0.0.0.31 access-list 198 permit ip any 203.0.113.96 0.0.0.31 access-list 198 permit ip any 192.168.100.80 0.0.0.15 ! Permisos "Igual con Igual" hacia Madrid (172.31.0.x) access-list 198 permit ip 198.51.100.0 0.0.0.31 172.31.0.0 0.0.0.31 access-list 198 permit ip 198.51.100.32 0.0.0.31 172.31.0.32 0.0.0.31 access-list 198 permit ip 198.51.100.64 0.0.0.31 172.31.0.64 0.0.0.31 access-list 198 permit ip 198.51.100.128 0.0.0.31 172.31.0.128 0.0.0.31 ! Permisos "Igual con Igual" hacia Buenos Aires (203.0.113.x) access-list 198 permit ip 198.51.100.0 0.0.0.31 203.0.113.0 0.0.0.31 access-list 198 permit ip 198.51.100.32 0.0.0.31 203.0.113.32 0.0.0.31 access-list 198 permit ip 198.51.100.64 0.0.0.31 203.0.113.64 0.0.0.31 access-list 198 permit ip 198.51.100.128 0.0.0.31 203.0.113.128 0.0.0.31 ! Permisos "Igual con Igual" hacia CDMX (192.168.100.x) access-list 198 permit ip 198.51.100.0 0.0.0.31 192.168.100.0 0.0.0.31 access-list 198 permit ip 198.51.100.32 0.0.0.31 192.168.100.32 0.0.0.31 access-list 198 permit ip 198.51.100.64 0.0.0.31 192.168.100.64 0.0.0.15 access-list 198 permit ip 198.51.100.128 0.0.0.31 192.168.100.96 0.0.0.15 ! Aplicación de la ACL 198 de salida interface Serial0/1/0 ip access-group 198 out exit interface Serial0/1/1 ip access-group 198 out exit interface Gigabit0/0/2 ip access-group 198 out exit do write crypto isakmp policy 10 authentication pre-share hash sha encryption aes 256 group 2 lifetime 86400 exit crypto isakmp key toor address 11.0.0.2 (IP del destino, 'toor' es la clave pre-compartida) crypto ipsec transform-set TSET esp-aes esp-sha-hmac ! ACL para definir el tráfico que se va a encriptar (Tráfico interesante) access-list 101 permit ip 192.168.1.0 0.0.0.255 192.168.11.0 0.0.0.255 access-list 101 permit ip 192.168.1.0 0.0.0.255 192.168.12.0 0.0.0.255 crypto map CMAP 10 ipsec-isakmp set peer 12.0.0.2 (IP de la interfaz serial pública del router destino) match address 101 (El número de la ACL de tráfico interesante) set transform-set TSET exit interface serial 0/1/0 crypto map CMAP do write exit Router2 interface Serial0/0/0 ip address 192.0.2.2 255.255.255.252 no shutdown interface Tunnel0 ip address 10.10.10.2 255.255.255.252 (IP del túnel) tunnel source Serial0/0/0 o IP (Interfaz o IP de salida pública) tunnel destination 192.0.2.1 (IP de la interfaz del lado destino) ! Enrutamiento sobre el túnel ip route 192.168.1.0 255.255.255.0 10.10.10.1 o sino con rip: router rip network 10.10.10.0 (en rip agregamos la red del túnel)